INTERNSHIP DAILY REPORT

Day 3  —  April 4, 2026

Topic: TMT Bars — Sizes, Weights, Bundle Charts & IS 1786 Tolerances

| Date | April 4, 2026 (Saturday) | Day No. | Day 3 |
| --- | --- | --- | --- |

| Department | Civil / Construction Materials | Standard Ref. | IS 1786 |
| --- | --- | --- | --- |

1. Objective of the Day

On Day 3 of the internship, the primary focus was to understand the physical and structural properties of TMT (Thermo-Mechanically Treated) bars used in construction. The trainee studied the standard sizes available in the market, how weight is calculated per meter and per bundle, bundle quantity for each diameter, and the tolerance limits permitted under the Bureau of Indian Standards specification IS 1786.

2. Topics Studied

2.1  TMT Bar Size & Weight Formula

The standard formula for calculating the weight of a TMT rod per metre is derived from its cross-sectional area and the density of iron:

Weight (kg/m)  =  (D² / 162)   where D = diameter in mm

Alternatively:  Weight = π × r² × Length × Density   (Density of iron = 0.00785 g/mm³)

The standard length of one TMT bar is 11 metres. Bundle weight is obtained by multiplying the weight per bar by the number of rods in the bundle.

3. TMT Bar — Size, Weight & Bundle Reference Chart

The following chart was studied and prepared based on the company's reference spreadsheet (Size_and_bundle_chart.xlsx). Density used: 0.00785 g/mm³; Standard bar length: 11 m.

| Size | Dia (mm) | Radius (mm) | Area (mm²) | Volume (cm³/m) | Wt/m (kg) | Qty/Bundle | Bundle Wt (kg) | IS 1786 Tolerance |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8mm | 8 | 4 | 50.24 | 0.3944 | 4.338 | 10 | 43.382 | ±7% |
| 10mm | 10 | 5 | 78.50 | 0.6162 | 6.778 | 7 | 47.449 | ±7% |
| 12mm | 12 | 6 | 113.04 | 0.8874 | 9.761 | 5 | 48.805 | ±5% |
| 16mm | 16 | 8 | 200.96 | 1.5775 | 17.353 | 3 | 52.059 | ±5% |
| 20mm | 20 | 10 | 314.00 | 2.4649 | 27.114 | 2 | 54.228 | ±3% |
| 25mm | 25 | 12.5 | 490.63 | 3.8514 | 42.365 | 1 | 42.365 | ±3% |
| 32mm | 32 | 16 | 803.84 | 6.3101 | 69.412 | 1 | 69.412 | ±3% |

Note: Bundle weight = (Weight per metre) × 11m × Quantity per bundle. Tolerance column reflects IS 1786 allowable mass variance.

4. Key Observations & Learnings

- The weight of a TMT bar increases significantly with diameter — a 32mm bar weighs approximately 16× more per metre than an 8mm bar.

- Bundle sizes decrease as diameter increases to keep the bundle weight practical for handling at the site (all bundles roughly between 42–50 kg).

- The IS 1786 standard serves as the regulatory backbone ensuring that TMT bars supplied across India meet minimum weight and quality benchmarks.

- The actual weight of a rod may vary slightly due to manufacturing processes, but it must remain within the specified percentage limits of the standard weight.

5. Activities Performed

- Studied and analyzed the Size_and_bundle_chart.xlsx spreadsheet containing TMT size, area, volume, weight per metre, bundle quantity, and bundle weight data.

- Researched IS 1786 tolerance standards for mass variance of TMT rods online and cross-referenced with site data.

- Understood the physical reasoning behind tolerance ranges for different bar sizes.

- Prepared detailed notes and compiled this internship report summarizing the day's learnings.

6. Conclusion

Day 3 provided a thorough understanding of TMT bar specifications critical to any construction project. Knowledge of standard bar weights, bundle quantities, and IS 1786 tolerances is essential for material procurement, quality checks, and structural calculations on site. This foundational understanding will be applied in subsequent internship days when studying reinforcement detailing and bar bending schedules.

Intern Signature: _______________________

Supervisor / Guide Signature: _______________________

Date: April 4, 2026